M C P R I N T

* * * * * * *

MCPRINT can be used to print from the Series 3 to a printer which is

connected to an IBM PC/XT/AT or compatible. The PC must have a free

serial port which is used to connect to the Series 3 via 3 Link. The

printer may be connected to a parallel or serial port on the PC.

Even if you can link the Series 3 directly to the printer, there may

be reasons why it is more convenient to use MCPRINT:

(1) The printer is shared by other PCs - either using a multi-port

printer buffer or a local area network - and it would be

unreasonable to connect it directly to the Series 3.

(2) You do not wish to disturb the connection between the PC and the

printer.

(3) You have already set up the serial connection to use MCLINK for

file transfer to the PC.

(4) When your PC is connected to more than one printer, you can

select which one the Series 3 will use.

Using MCPRINT

-------------

Physically connect the Series 3 to the PC with 3 Link - exactly as

for MCLINK. If the printer is connected to LPT1, and 3 Link is

connected to COM1, you can now run MCPRINT on the PC by typing:

MCPRINT

If LPT1 and COM1 are not the ports used, parameters are required,

as described below.

Exiting MCPRINT

~~~~~~~~~~~~~~~

To exit MCPRINT, press Control-C.

Printer configuration on the Series 3

-------------------------------------

On the Series 3, select the Printer Setup option from the Special

menu in the System screen, to display the `Printer configuration'

dialog. Set the `Printer device' line to `Serial'. You don't normally

have to change the `Serial characteristics' (it displays a subdialog

when you press Tab) because MCPRINT uses the default settings.

You can now print as if the printer was directly connected to the

Series 3 - by selecting the `Print' option in any application which

can print - including the built-in Word Processor, Agenda, Database,

and Program editor. (Remember first to use the `Print setup' options

in these applications, to tell the Series 3 about the type of printer

and the page layout desired.) As far as the Series 3 software is

concerned, it is printing to a serial printer.

Parameters

----------

MCPRINT takes the following parameters:

<prdev> -c<port> -t<timeout> -q

all of which are optional. To be reminded of these parameters, type:

MCPRINT ?

The <prdev> parameter

~~~~~~~~~~~~~~~~~~~~~

<prdev> is the print device name, as for the DOS PRINT command. If

omitted, the default is LPT1 (the DOS name for the first parallel

port).

For example, to print to LPT2, type:

MCPRINT LPT2

If the PC is a station on a local area network, LPT2, LPT3 etc. may be

used to redirect output to remote printers attached to the network

server.

The <prdev> parameter may specify any suitable output device. If you

have a printer connected to a second serial port on the PC, you can

use:

MCPRINT COM2

In this case, you should have previously used the DOS MODE command to

set the serial parameters to be used between the PC and the printer.

You can also print to a file on the PC using, for example:

MCPRINT PRINT.LIS

Note that PRINT.LIS will be overwritten each time you print.

To test the connection without wasting paper, type:

MCPRINT CON

CON is the DOS name for the console (screen). When you then print

from the Series 3, you should see the output appear on the screen of

the PC.

The -c<port> parameter

~~~~~~~~~~~~~~~~~~~~~~

<-c> is the serial port on the PC to which 3 Link is connected. If

omitted, the default is port 1, which corresponds to COM1. If 3 Link

is connected to the PC's second serial port, type:

MCPRINT -C2

The -t<timeout> parameter

~~~~~~~~~~~~~~~~~~~~~~~~~

<timeout> is a number of seconds. This parameter is provided for use

on local area networks where it is necessary to close and open the

print device between each print job. It is used to specify an

inactivity time-out in seconds. If there is no printing for this

period, the print device is automatically closed. For example:

MCPRINT LPT2 -T5

will close the print device after 5 seconds of inactivity.

If the parameter is omitted, the print device is not automatically

closed. You don't have to specify a time-out, as you can close the

print device manually by pressing any key on the PC keyboard. Exiting

MCPRINT will also close the print device.

The Series 3 is multi-tasking - while one application is printing,

you can carry on with something else. However, the background

printing can be held up at times, depending on the processing

requirements of the work you are doing. This can fool MCPRINT's

inactivity time-out into thinking that the printing has finished,

causing it to prematurely close the print device. If you experience

this problem, consider increasing the time-out, or convert to closing

the printer device manually by pressing a key on the PC keyboard.

The -q parameter

~~~~~~~~~~~~~~~~

This parameter suppresses status messages (the q stands for "quiet").